# Required Components					    BOM

|  | Thumbstick without Press Down Function - 1x PS2 Joystick - 12x Dupont Wires: 4 male/male, 4 male/female, 4 female/female – it is recommended that the same 4 colours are used for the different types of wires as they will be getting connected. - 1x Headphone Adapter - 8x #4 1/2" screws | Thumbstick with Press Down Function - 1x PS2 Joystick - 16x Dupont Wires: 6 male/male, 5 male/female, 5 female/female – it is recommended that the same 5 colours are used for the different types of wires as they will be getting connected. - 2x Headphone Adapters - 8x #4 1/2" screws |
| --- | --- | --- |

# Required Tools

- Small flat headed screwdriver

- Tape

## Assembly Instructions for the Analog Thumbstick without the Press Down Function – pg 2

## Assembly Instructions for the Analog Thumbstick with the Press Down Function – pg 5

Please refer to the Analog Joystick Maker Checklist to ensure you ask the user which version of the joystick they want.

# Assembly Instructions for Analog Thumbstick without the Press Down Function

## Step 1

Male-Male

Locate 12 dupont wires: 4 male – female, 4 male-male, 4 female-female. It is also recommended that you use the same colours for each of these different types of wires.

Male-Female Female-Female

## Step 2

Connect the same colour wires so that you now have wires 3 lengths long, with one male end and one female end.

Female Male

## Step 3

Use tape to secure the connected wires.

## Step 4

Feed the female end of the wires through the 3 holes in the 3D printed joystick base.

## Step 5

Connect the wires to the GND, +5V, VRX, and VRY prongs on the joystick.

## Step 6

Connect the joystick to the 3D printed base with 4 screws.

## Step 7 Loosen the 4 screws on the headphone adapter.

## Step 8

Insert the male ends of the wires under the screws in the headphone adapter according to the diagram above. When inserted, tighten the screws to secure the wires.

## Step 9

This is what the device should look like thus far.

## Step 10

Align the 3D printed top of the joystick with the base.

## Step 11

Insert 4 screws in the base of the joystick to hold the top and base together.

# Assembly Instructions for Analog Thumbstick with the Press Down Function

## Step 1

Male-Male

Locate 12 dupont wires: 4 male – female, 4 male-male, 4 female-female. It is also recommended that you use the same colours for each of these different types of wires.

Male-Female Female-Female

## Step 2

Connect the same colour wires so that you now have wires 3 lengths long, with one male end and one female end.

Female Male

## Step 3

Use tape to secure the connected wires.

## Step 4

Locate 3 more dupont wires: 1 male – female, 1 male-male, 1 and 1 female-female of the same colour. Connect and tape them to each other similar to steps 2 and 3.

## Step 5

Feed the female end of the 5 wires through the 3 holes in the 3D printed joystick base.

## Step 6

Connect the group of 4 wires to the GND, +5V, VRX, and VRY prongs on the joystick. Connect the single 5th wire to the SW prong.

## Step 7

Connect the joystick to the 3D printed base with 4 screws.

## Step 8

Loosen the 4 screws on both of the headphone adapters.

## Step 9

Insert the male ends of the wires under the screws in the headphone adapters according to the diagram above. This will also require a single male-male Dupont wire to be used as shown above in addition to previously mentioned wires. When inserted, tighten the screws to secure the wires. Note: on headphone adapter 1, 2 wires will be going into the V port. This may be challenging but both wires should be able to be secured with one screw.

Wires from Step 2 Wire from Step 4 Single male-male Dupont wire

## Step 10

This is what the device should look like thus far.

## Step 11

Align the 3D printed top of the joystick with the base.

## Step 12

Insert 4 screws in the base of the joystick to hold the top and base together.